SMART WASTE MANAGEMENT SYSTEM FOR METROPOLITAN CITIES

Prepared By

Name: Balaji Tanaji Patil
PRN No: 2022011031141
Roll No: 62

1. ABSTRACT

Rapid urbanization and increasing population in metropolitan cities have significantly increased the volume of municipal solid waste. Traditional waste collection methods often lead to overflowing bins, delayed collection schedules, environmental pollution, and inefficient resource utilization.

The Smart Waste Management System for Metropolitan Cities is an IoT-enabled intelligent waste monitoring solution designed to optimize waste collection operations through real-time monitoring and predictive analytics. The system utilizes an ESP32-based smart waste bin equipped with multiple sensors including ultrasonic sensors, gas sensors, temperature and humidity sensors, PIR motion sensors, solar charging modules, and battery monitoring systems.

Real-time sensor data is transmitted through Node-RED APIs and visualized on a MERN Stack Dashboard. The dashboard provides monitoring of waste fill levels, methane gas concentration, battery status, environmental conditions, and collection alerts. The system enables municipal authorities to efficiently manage waste collection routes, reduce operational costs, improve sanitation, and support sustainable smart city initiatives.

2. OBJECTIVES

The primary objectives of the project are:

- To monitor garbage bin fill levels in real-time.

- To detect harmful methane and toxic gases generated from waste.

- To provide automated alerts when bins reach critical capacity.

- To monitor environmental conditions such as temperature and humidity.

- To implement solar-powered charging for sustainable operation.

- To visualize IoT telemetry through an interactive MERN dashboard.

- To optimize waste collection schedules and routes.

- To reduce urban pollution and improve city cleanliness.

3. SYSTEM ARCHITECTURE

The proposed system consists of four major layers:

A. Hardware Layer

- ESP32 NodeMCU

- HC-SR04 Ultrasonic Sensor

- MQ-135 Gas Sensor

- DHT11 Temperature & Humidity Sensor

- PIR Motion Sensor

- SG90 Servo Motor

- OLED Display

- LCD Display

- Buzzer

- Status LEDs

- Solar Panel

- Battery Monitoring Unit

B. Communication Layer

- Wi-Fi Connectivity

- REST API Communication

- Node-RED Integration

C. Application Layer

- React.js Frontend

- Node.js Backend

- Express.js APIs

- MongoDB Database

D. Visualization Layer

- Smart Dashboard

- Hardware Simulator

- Analytics Charts

- Alert System

4. HARDWARE COMPONENTS USED

| Sr No | Component | Purpose |
| --- | --- | --- |
| 1 | ESP32 NodeMCU | Main Microcontroller |
| 2 | HC-SR04 Ultrasonic Sensor | Measure Bin Fill Level |
| 3 | MQ-135 Gas Sensor | Detect Methane & Toxic Gases |
| 4 | DHT11 Sensor | Temperature & Humidity Monitoring |
| 5 | PIR Motion Sensor | Human Detection |
| 6 | SG90 Servo Motor | Automatic Lid Control |
| 7 | SSD1306 OLED Display | Live Status Visualization |
| 8 | 16x02 LCD Display | Data Display |
| 9 | Piezo Buzzer | Alert Generation |
| 10 | LEDs | Status Indication |
| 11 | Solar Panel | Battery Charging |
| 12 | Rechargeable Battery | Power Supply |

5. ESP32 PIN CONNECTIONS

| Component | ESP32 Pin |
| --- | --- |
| LCD SDA | GPIO 21 |
| LCD SCL | GPIO 22 |
| OLED SDA | GPIO 21 |
| OLED SCL | GPIO 22 |
| Ultrasonic Trig | GPIO 5 |
| Ultrasonic Echo | GPIO 18 |
| MQ-135 AO | GPIO 34 |
| DHT11 Data | GPIO 33 |
| Servo PWM | GPIO 39 |
| PIR OUT | GPIO 15 |
| Buzzer | GPIO 25 |
| Green LED | GPIO 12 |
| Yellow LED | GPIO 14 |
| Red LED | GPIO 27 |
| Blue LED | GPIO 26 |

6. HARDWARE SIMULATOR MODULE

The project includes an advanced virtual hardware simulator embedded inside the MERN dashboard. The simulator replicates the actual ESP32 IoT circuit and allows users to interact with virtual sensors.

Features

6.1 OLED Display

- Smart Waste Bin Visualization

- Fill Level Indicator

- Wi-Fi Signal Strength

- Battery Status

- Gas Sensor Readings

- Humidity Monitoring

6.2 Servo Controlled Smart Lid

- Automatic lid opening

- Motion-triggered operation

- 90-degree servo rotation

6.3 PIR Motion Detection

- Human presence detection

- Automatic waste disposal assistance

- Sensor activity indication

6.4 Solar Charging System

- Real-time charging simulation

- Battery monitoring

- Sustainable energy utilization

6.5 Smart Alarm System

- Overflow alerts

- Toxic gas alerts

- Visual and audio notifications

7. TELEMETRY DATA STRUCTURE

The simulator continuously synchronizes data with the backend using REST APIs.

{

"Bin_ID": "BIN001",

"Fill_Level": 75,

"Methane_PPM": 120,

"Temperature": 28,

"Humidity": 55,

"Battery_Level": 85,

"Lid_Status": "CLOSED",

"Solar_Charge": 0.00

}

8. SOFTWARE REQUIREMENTS

Frontend

- React.js

- Tailwind CSS

- Axios

- Recharts

Backend

- Node.js

- Express.js

- Node-RED

Database

- MongoDB

Development Tools

- VS Code

- GitHub

- Postman

- Wokwi Simulator

9. WORKING METHODOLOGY

Step 1

Ultrasonic sensor measures garbage level.

Step 2

MQ-135 detects methane gas concentration.

Step 3

DHT11 records temperature and humidity.

Step 4

PIR sensor detects nearby users.

Step 5

Servo motor opens waste bin lid automatically.

Step 6

Sensor data is transmitted through Wi-Fi.

Step 7

Node-RED processes incoming telemetry.

Step 8

Dashboard displays real-time analytics.

Step 9

Alerts are generated for:

- Full Bin

- Toxic Gas

- Low Battery

10. ADVANTAGES

- Real-time monitoring

- Reduced operational costs

- Efficient waste collection

- Smart city integration

- Environment-friendly solution

- Renewable solar charging

- Improved sanitation

- Automated alerts

11. APPLICATIONS

- Smart Cities

- Municipal Corporations

- Railway Stations

- Airports

- Hospitals

- Educational Institutions

- Shopping Malls

- Industrial Areas

12. FUTURE SCOPE

- AI-based waste prediction.

- Route optimization using Machine Learning.

- GPS-enabled garbage trucks.

- Mobile application integration.

- Smart analytics and forecasting.

- Image-based waste classification using AI.

- Cloud-based monitoring systems.

13. CONCLUSION

The Smart Waste Management System for Metropolitan Cities provides an intelligent, scalable, and sustainable solution for urban waste management. Through IoT-enabled monitoring, real-time telemetry, automated alerts, and interactive dashboard visualization, the system improves collection efficiency, reduces environmental hazards, and supports smart city initiatives. The integration of ESP32 hardware, Node-RED communication, and MERN Stack technologies creates a complete end-to-end waste management ecosystem suitable for modern metropolitan infrastructure.

REFERENCES

- ESP32 NodeMCU Documentation

- HC-SR04 Ultrasonic Sensor Datasheet

- MQ-135 Gas Sensor Datasheet

- DHT11 Sensor Documentation

- Node-RED Official Documentation

- MongoDB Documentation

- React.js Documentation

- Smart City Waste Management Research Papers